HemoScan AI

Non-Invasive Hemoglobin Monitor

Technical Documentation & System Architecture

| Project Name | HemoScan AI — Non-Invasive Hemoglobin Monitor |
| --- | --- |
| Team | Medicore Innovation |
| Members | Mahambihai T (Founder & Tech Lead), Sujay B (CEO), Xavier Jerfin (CXO) |
| Institution | St. Michael College of Engineering & Technology, Kalayarkoil |
| Hackathon | Bharat Academix CodeQuest 2026 — Round 2 Submission |
| Submission Date | June 2026 |
| Version | MVP v1.0 |
| Registration | Udyam Certified Startup ¦ Startup TN Recognized ¦ IJERT Published |

# 1. Executive Summary

HemoScan AI is a non-invasive hemoglobin monitoring solution developed by Medicore Innovation. The system uses multi-wavelength LED photoplethysmography (PPG) combined with Beer-Lambert Law signal processing and AI-based calibration to estimate hemoglobin concentration without drawing blood.

India has over 500 million anemia patients, most of whom cannot afford frequent lab-based CBC tests costing ₹200–₹500 per visit. HemoScan AI reduces the cost to under ₹2 per test, eliminates needle use, and delivers results in under 30 seconds — making hemoglobin monitoring accessible to ASHA workers, rural PHCs, and home patients.

This document covers the complete system architecture, algorithm design, software implementation, hardware specifications, and deployment strategy for the MVP submitted for Round 2.

# 2. Problem Statement

## 2.1 Current Challenges

- Invasive CBC blood tests require needle pricks every visit — painful, infection risk, and feared by patients

- Lab tests cost ₹200–₹500 per visit; frequent monitoring is financially inaccessible for rural families

- 80% of India's 500M+ anemia patients go undetected due to lack of rural diagnostic infrastructure

- Pregnant women, children, and elderly face highest risk — with no affordable point-of-care screening

- Existing pulse oximeters measure SpO2 only — they cannot measure hemoglobin concentration

## 2.2 Market Gap

No consumer-grade, affordable, non-invasive hemoglobin monitor exists in India today. Medical-grade devices (like Masimo Pronto) cost ₹80,000+. HemoScan AI targets the ₹1,499 consumer price point with a ₹2/test operating cost — a 150x cost reduction.

# 3. Solution Overview — HemoScan AI

## 3.1 Core Technology

HemoScan AI uses Photoplethysmography (PPG) — the same optical principle used in pulse oximeters — extended to 4 wavelengths for hemoglobin-specific absorption measurement.

| Wavelength | Purpose |
| --- | --- |
| 660nm (Red) | Oxygenated hemoglobin (HbO2) peak absorption |
| 880nm (Near-IR) | Deoxygenated hemoglobin (Hb) absorption |
| 520nm (Green) | SpO2 baseline and motion artifact reference |
| 940nm (IR-2) | Calibration channel and deep tissue reference |

## 3.2 Beer-Lambert Law Algorithm

The core hemoglobin calculation follows Beer-Lambert Law: Absorbance = ε × c × l, where ε is the molar absorption coefficient, c is the hemoglobin concentration, and l is the optical path length through tissue.

Our implementation computes the AC (pulsatile) and DC (baseline) components of each wavelength channel. The Ratio of Ratios (R) is calculated as:

R = (AC_red / DC_red) / (AC_ir / DC_ir)

Hemoglobin concentration is then derived using empirically-fitted coefficients:

Hb (g/dL) = a - (b × R)    [where a = 22.0, b = 6.5 for demo calibration]

Note: Coefficients above are demo-calibrated. Clinical validation (Phase 3) will use hospital-derived regression coefficients across diverse skin tones and patient demographics.

# 4. System Architecture

## 4.1 Hardware Layer

| Component | Specification |
| --- | --- |
| Microcontroller | ESP32-S3 (Dual-core 240MHz, WiFi/BT, 512KB SRAM) |
| PPG Sensor | MAX30102 (Red 660nm + IR 880nm channels, 18-bit ADC) |
| Additional LEDs | 520nm Green LED + 940nm IR LED (Phase 2 hardware) |
| Display | OLED SSD1306 128x64px (I2C interface) |
| Power | Li-Po 3.7V 1000mAh, USB-C charging, ~8hr runtime |
| Casing | Silicone ergonomic finger clip (Phase 2 fabrication) |
| PCB | Custom 4-layer PCB design (Phase 2) |
| Current MVP | Breadboard prototype with ESP32 DevKit + MAX30102 breakout |

## 4.2 Software Layer

| Layer | Technology Stack |
| --- | --- |
| Firmware | Arduino C++ on ESP32, 50Hz PPG sampling, UART output |
| Signal Processing | Beer-Lambert Law, AC/DC decomposition, Kalman filtering |
| Web Dashboard | Vanilla HTML5 + CSS3 + JavaScript (no framework dependency) |
| Camera PPG | WebRTC getUserMedia API, red-channel pixel analysis |
| Data Persistence | Browser localStorage, session-based patient records |
| Simulation | In-browser ESP32 firmware logic model (JavaScript) |
| Charts | HTML5 Canvas (native, no external library) |

## 4.3 Data Flow

- Finger placed on sensor (hardware) or camera lens (mobile MVP)

- 4-wavelength LED array fires sequentially at 50Hz sampling rate

- Photodiode (or camera sensor) captures reflected light intensity per wavelength

- 100-sample buffer (~2 seconds) accumulated; AC/DC components separated

- Beer-Lambert Law R-value calculated; Hb concentration estimated

- Result displayed on OLED (hardware) or web dashboard (MVP); patient record saved

# 5. MVP Deliverables — Round 2

## 5.1 Functional Prototype / MVP

The MVP is a single self-contained HTML5 web application (hemoscan_complete.html) that demonstrates the complete HemoScan AI user flow:

- Camera-based PPG scan using device rear camera and torch/flash

- Real-time red-channel pixel analysis implementing Beer-Lambert Law

- PPG waveform visualization using HTML5 Canvas

- Hemoglobin result display with status classification (Normal / Mild Anemia / Severe Anemia)

- Patient details form (Name, Age, Gender, Village) post-scan

- Persistent patient dashboard with trend chart and AI health recommendations

- Patient management portal with individual history tracking

- Works on any modern mobile browser via CodePen HTTPS deployment

## 5.2 ESP32 Firmware Simulation

A complete Arduino sketch (hemoscan_sketch.ino) implements the embedded logic including MAX30102 initialization, 100-sample PPG buffer acquisition, Beer-Lambert calculation, and Serial Monitor output — validated via Wokwi circuit simulation.

## 5.3 Embedded Logic Browser Demo

hemoscan_simulation.html demonstrates the exact ESP32 firmware logic running in-browser, with LED sequence animation, real-time signal display, progress tracking, and hemoglobin result output — providing a working software model of the target hardware.

# 6. AI & Intelligence Features

## 6.1 Signal Processing AI

- AC/DC decomposition for pulsatile signal extraction

- Motion artifact filtering (standard deviation gating)

- Skin tone compensation through ratio-based measurement (reduces melanin interference)

- Adaptive baseline correction across different lighting conditions

## 6.2 Clinical Intelligence

- Automatic hemoglobin status classification (WHO anemia thresholds)

- Trend analysis across multiple readings — detects improving/declining patterns

- Personalized AI health recommendations based on latest Hb level

- Risk scoring for anemia severity (Critical / Low / Normal)

- Population-level anemia heatmap analytics for district health planning

## 6.3 Future AI Roadmap

- CNN model trained on 50,000+ PPG signals for improved accuracy

- Federated learning for continuous model improvement across devices

- Integration with national health records (ABHA ID)

- Predictive anemia risk scoring using longitudinal patient data

# 7. Accuracy & Validation Status

| Metric | Current Status |
| --- | --- |
| Algorithm Type | Beer-Lambert Law (demo-calibrated coefficients) |
| MVP Accuracy | Demo-grade: ±1.5–2.0 g/dL variation (camera PPG) |
| Firmware Accuracy | Simulation-validated; physical sensor testing pending |
| Clinical Validation | Phase 3 roadmap — hospital partnership required |
| Target Accuracy | ±0.5 g/dL (comparable to SpO2 pulse oximeters) |
| Skin Tone Support | Ratio-based algorithm reduces melanin interference |
| Regulatory Status | Pre-clinical; CDSCO submission planned Phase 3 |

Disclaimer: This MVP is a research prototype for demonstration purposes. Readings should not be used for medical diagnosis. Clinical-grade accuracy requires hospital validation with calibrated reference CBC equipment.

# 8. Scalability & Architecture

## 8.1 Hardware Scalability

- Phase 2: PCB miniaturization to fingertip clip form factor (<30g)

- Phase 3: USB-C charging, Bluetooth data sync to mobile app

- Phase 4: IoT cloud connectivity for real-time health data upload

## 8.2 Software Scalability

- Current MVP: Client-side only (no server required), works offline

- Phase 2: FastAPI + PostgreSQL backend for multi-user data management

- Phase 3: TimescaleDB for time-series health analytics at scale

- Phase 4: District-level government dashboard, ASHA network integration

## 8.3 Market Scalability

- B2G: Government PHC contracts — 250,000 PHCs across India

- B2B: Hospital chains, diagnostic labs, corporate health programs

- B2C: Direct-to-consumer device at ₹1,499 price point

- Subscription model: ₹2/scan for consumables/calibration service

# 9. Project Roadmap

| Phase | Timeline | Deliverables |
| --- | --- | --- |
| Phase 1 ✅ | Complete | Beer-Lambert algorithm design, web dashboard MVP, camera PPG implementation, ESP32 firmware code, IJERT published paper, startup registration |
| Phase 2 | 3 months | Breadboard prototype build, PCB fabrication, silicone casing, OLED integration, 4-wavelength LED array, battery optimization |
| Phase 3 | 6 months | Clinical validation, hospital testing, accuracy benchmarking, CDSCO regulatory submission |
| Phase 4 | 12 months | Market launch, rural distribution, mobile app, government PHC contracts, ASHA network integration |

# 10. Team — Medicore Innovation

| Name | Role ¦ Contribution |
| --- | --- |
| Mahambihai T | Founder & Tech Lead | Hardware design, Beer-Lambert algorithm, web dashboard, camera PPG implementation, IJERT paper |
| Sujay B | Co-Founder & CEO | Business strategy, market research, investor relations, partnership development |
| Xavier Jerfin | Co-Founder & CXO | Technical guidance, clinical domain expertise, faculty mentorship, institutional support |

## 10.1 Startup Credentials

- Udyam Registered Startup (MSME certified)

- Startup TN Recognized — Tamil Nadu Government

- IJERT Published Research Paper — Vol. 15 Issue 03

- Institution: St. Michael College of Engineering & Technology, Kalayarkoil, Sivagangai

# 11. Disclaimer

This document describes a research prototype submitted for the Bharat Academix CodeQuest 2026 hackathon. HemoScan AI is currently at Technology Readiness Level (TRL) 4-5. The MVP demonstrates working software logic and algorithm implementation. Physical hardware integration and clinical validation are planned for Phase 2 and Phase 3 respectively.

All hemoglobin readings from the MVP camera-based system are demo-grade estimates and must not be used for medical diagnosis or treatment decisions. Clinical-grade accuracy requires calibration with reference CBC laboratory equipment and approval from CDSCO/regulatory authorities.

© 2026 Medicore Innovation. All Rights Reserved.